Problem 1.6

Henri Darcy, a French engineer, proposed that the pressure drop 𝛥p for flow at velocity V through a tube of length L could be correlated in the form

∆pρ   =   α  L V2

If Darcy’s formulation is consistent, what are the dimensions of the coefficient  α?