Problem 1.8

Suppose we know little about the strength of materials but are told that the bending stress σ in a beam is proportional to the beam half-thickness y and also depends on the bending moment M and the beam area moment of inertia I . We also learn that, for the particular case M  2900 inlbf, y  1.5 in, and I  0.4 in4, the predicted stress is 75 MPa. Using this information and dimensional reasoning only, find, to three significant figures, the only possible dimensionally homogeneous formula σy f(M,I).