Problem 1.27

For steam at a pressure of 45 atm, some values of temperature and specific volume are as follows, from Ref. 23:

| T, ºF | 500 | 600 | 700 | 800 | 900 |
| --- | --- | --- | --- | --- | --- |
| v, ft3/lbm | 0.7014 | 0.8464 | 0.9653 | 1.074 | 1.177 |

Find an average value of the predicted gas constant R in m2/(s2∙K).   Does this data reasonably approximate an ideal gas?  If not, explain.