Problem 1.34

Consider steam at the following state near the saturation line: (p1, T1)  (1.31 MPa, 290°C). Calculate and compare, for an ideal gas (Table A.4) and the steam tables (a) the density 1; and (b) the density 2 if the steam expands isentropically to a new pressure of 414 kPa. Discuss your results.