Problem 1.50

A simple viscometer measures the time t for a solid sphere to fall a distance L through a test fluid of density . The fluid viscosity  is then given by

where D is the sphere diameter and Wnet is the sphere net weight in the fluid.

(a) Prove that both of these formulas are dimensionally homogeneous.

(b) Suppose that a 2.5 mm diameter aluminum sphere (density 2700 kg/m3) falls in an oil of density 875 kg/m3. If the time to fall 50 cm is 32 s, estimate the oil viscosity and verify that the inequality is valid.