Problem 1.59

A solid cylinder of diameter D, length L, density s falls due to gravity inside a tube of diameter Do. The clearance,  is filled with a film of viscous fluid (,).

Derive a formula for terminal fall velocity and apply to SAE 30 oil at 20C for a steel cylinder with D  2 cm, Do  2.04 cm, and L  15 cm. Neglect the effect of any air in the tube.