Problem 1.60

Pipelines are cleaned by pushing through them a close-fitting cylinder called a pig.  The name comes from the squealing noise it makes sliding along.  Ref. 50 describes a new non-toxic pig, driven by compressed air, for cleaning cosmetic and beverage pipes.   Suppose the pig diameter is 5-15/16 in and its length 26 in.  It cleans a 6-in-diameter pipe at a speed of 1.2 m/s.  If the clearance is filled with glycerin at 20C, what pressure difference, in pascals, is needed to drive the pig?  Assume a linear velocity profile in the oil and neglect air drag.