Problem 1.C11

Mott [Ref. 49, p. 38] discusses a simple falling-ball viscometer, which we can analyze later in Chapter 7.  A small ball of diameter D and density b falls through a tube of test liquid (.

The fall velocity V is calculated by the time to fall a measured distance.    The formula for calculating the viscosity of the fluid is

This result is limited by the requirement that the Reynolds number (VD/) be less than 1.0.

Suppose a steel ball (SG = 7.87) of diameter 2.2 mm falls in SAE 25W oil (SG = 0.88) at 20C.  The measured fall velocity is 8.4 cm/s.  (a) What is the viscosity of the oil, in kg/m-s?

(b) Is the Reynolds number small enough for a valid estimate?