Problem 1.C2

When a person ice-skates, the ice surface actually melts beneath the blades, so that he or she skates on a thin film of water between the blade and the ice.

(a) Find an expression for total friction force F on the bottom of the blade as a function of skater velocity V, blade length L, water film thickness h, water viscosity , and blade width W.

( b ) Suppose an ice skater of total mass m is skating along at a constant speed of V 0 when she suddenly stands stiff with her skates pointed directly forward, allowing herself to coast to a stop. Neglecting friction due to air resistance, how far will she travel before she comes to a stop? (Remember, she is coasting on two skate blades.) Give your answer for the total distance traveled, x , as a function of (Vo, m, L, h, , W).

(c) Compute x for the case Vo  4 m/s, m  100 kg, L  30 cm, W  5.0 mm, and h  0.10 mm. Do you think our assumption of negligible air resistance is a good one?