Problem 1.C3

Two thin flat plates, tilted at an angle α , are placed in a tank of liquid of known surface tension ϒ and contact angle θ , as shown in Fig. C1.3. At the free surface of the liquid in the tank, the two plates are a distance L apart and have width b into the page. The liquid rises a distance h between the plates, as shown.

( a ) What is the total upward ( z -directed) force, due to surface tension, acting on the liquid column between the plates?

( b ) If the liquid density is ρ , find an expression for surface tension ϒ in terms of the other variables.