Problem 1.C4

Oil of viscosity  and density  drains steadily down the side of a tall, wide vertical plate,

as shown in Fig. C1.4. In the region shown, fully developed conditions exist; that is, the velocity profile shape and the film thickness d are independent of distance z along the plate. The vertical velocity w becomes a function only of x , and the shear resistance from the atmosphere is negligible.

( a ) Sketch the approximate shape of the velocity profile w(x), considering the boundary conditions at the wall and at the film surface.

( b ) Suppose film thickness δ , and the slope of the velocity profile at the wall, (dw / dx)wall , are measured by a laser Doppler anemometer (to be discussed in Chap. 6). Find an expression for the viscosity of the oil as a function of ρ , δ , (dw / dx)wall , and the gravitational acceleration g . Note that, for the coordinate system given, both w and (dw / dx)wall are negative.