Problem 1.C5

Viscosity can be measured by flow through a thin-bore or capillary tube if the flow rate is low. For length L, (small) diameter  pressure drop p, and (low) volume flow rate Q, the formula for viscosity is  = D4p/(CLQ), where C is a constant.

(a) Verify that C is dimensionless. The following data are for water flowing through a 2-mm-diameter tube which is 1 meter long. The pressure drop is held constant at p = 5 kPa.

| T, °C: | 10.0 | 40.0 | 70.0 |
| --- | --- | --- | --- |
| Q, L/min: | 0.091 | 0.179 | 0.292 |

(b) Using proper SI units, determine an average value of C by accounting for the variation with temperature of the viscosity of water.