Problem 1.C8

A mechanical device that uses the rotating cylinder of Fig. C1.6 is the Stormer viscometer [29]. Instead of being driven at constant Ω , a cord is wrapped around the shaft and attached to a falling weight W . The time t to turn the shaft a given number of revolutions (usually fi ve) is measured and correlated with viscosity. The formula is

where A and B are constants which are determined by calibrating the device with a known fluid. Here are calibration data for a Stormer viscometer tested in glycerol, using a weight of 50 N:

| , kg/m·s: | 0.23 | 0.34 | 0.57 | 0.84 | 1.15 |
| --- | --- | --- | --- | --- | --- |
| t, sec: | 15 | 23 | 38 | 56 | 77 |

(a) Find reasonable values of A and B to fit this calibration data. [Hint: The data are not very sensitive to the value of B.]

(b) A more viscous fluid is tested with a 100 N weight and the measured time is 44 s. Estimate the viscosity of this fluid.