Solution 1.4

Granular materials do indeed flow, at a rate that can be measured by “flowmeters”.  But they are not true fluids, because they can support a small shear stress without flowing.  They may rest at a finite angle without flowing, which is not possible for liquids (see Prob. P1.3). The maximum such angle, above which sand begins to flow, is called the angle of repose.  A familiar example is sugar, which pours easily but forms a significant angle of repose on a heaping spoonful.  The physics of granular materials are complicated by effects such as particle cohesion, clumping, vibration, and size segregation.  See Ref. 54 to learn more.