Solution 1.9

First find the volume of the liquid in m3:

Hemisphere volume =  12π6 D3  = 12π6(26 in)3 =  4601 in3÷61024in3m3=  0.0754m3

Liquid mass =  1617 oz ÷16 =  101 lbm 0.45359kglbm=   45.84 kg

Then the liquid density = 45.84 kg0.0754 m3 =  607kgm3     Ans.(a)

From Appendix Table A.3, this could very well be ammonia.    Ans.(b)