Solution 1.14

So far we know that Q B fcn(H,g). Write this in dimensional form:

Fig. P1.14

So the function fcn(H,g) must provide dimensions of {L2/T}, but only g contains time. Therefore g must enter in the form g1/2 to accomplish this. The relation is now

Q  Bg1/2fcn(H), or: {L3/T}  {L}{L1/2/T}{fcn(H)}, or: {fcn(H)}  {L3/2}In order for fcn(H) to provide dimensions of {L3/2}, the function must be a 3/2 power. Thus the final desired homogeneous relation for dam flow is:

Q  C B g1/2 H3/2, where C is a dimensionless constant Ans.