Solution 1.17

Write out the dimensions of each side of the equation:

The constant 61.9 has fractional dimensions: {61.9}  {L1.45T0.08M–0.54} Ans.

Clearly, the formula is extremely inconsistent and cannot be used with confidence for any given fluid or condition or units. Actually, the Hazen-Williams formula, still in common use in the water supply industry, is valid only for water flow in smooth pipes larger than 2-in. diameter and turbulent velocities less than 10 ft/s and (certain) English units. This formula should be held at arm’s length and given a vote of “No Confidence.”