Solution 1.25

This is a problem in handling awkward units.  Even if we use the BG system, we have to convert.  But, since the problem calls for a metric result, better we should convert to SI units:

T  =  74ºF  +  460   =   534ºR  x 0.5556  (inside front cover)    =   297 K

p   =   14.5 lbf/in2 x 6894.8  (inside front cover)   =   100,700 Pa

The SI gas constant, from Eq. (1.12), is 287 m2/(s2∙K).  Thus, from the ideal gas law, Eq. (1.10),

ρ =   pRT =  100,700 N/m2(287m2s2∙K)(297 K) =   1.18 N∙s2 m4

This doesn’t look like a density unit, until we realize that 1 N ≡ 1 kg∙m/s2.  Making this substitution, we find that

ρ   =     1.18  kg/m3                                 Answer

Notice that faithful use of SI units will lead to faithful SI results, without further conversion.