Solution 1.27

If ideal, the calculated gas constant would, from Table A.4, be about 461 m2/(s2∙K).  Try this for the first value, at T = 500ºF = 960ºR.  Change to SI units, using the inside front cover conversions:

T = 500ºF = 960ºR ÷ 1.8  = 533 K;     p = 45(101,350 Pa)  =  4561 kPa

v = 0.7014 ft3/lbm ÷ 16.019  = 0.0438 m3/kg

p v=R T , or:  R= p vT= 4.561E6 Pa(0.0438 m3/kg)533 K  =  374 m2s2∙K

This is about 19% lower than the recommended value of  Rsteam  =  461 m2/(s2∙K).  Continue by filling out the rest of the table:

| T, ºF | 500 | 600 | 700 | 800 | 900 |
| --- | --- | --- | --- | --- | --- |
| R, m2/(s2∙K) | 374 | 409 | 427 | 437 | 443 |

These are all low, with an average of 418, nine per cent low.   The temperature is too low and the pressure too high.  We are too near the saturation line for the ideal gas law to be accurate.