Solution 1.35

In elementary kinetic theory of gases [21], k is related to the number of “degrees of freedom” of the gas: k  1  2/N, where N is the number of different modes of translation, rotation, and vibration possible for the gas molecule.

Example: Monotomic gas, N  3 (translation only), thus k  5/3

This explains why helium and argon, which are monatomic gases, have k  1.67.

Example: Diatomic gas, N  5 (translation plus 2 rotations), thus k  7/5

This explains why air, nitrogen, oxygen, NO, CO and hydrogen have k  1.40.

But NH3 has four atoms and therefore more than 5 degrees of freedom, hence k will be less than 1.40.  Most tables list k = 1.32 for NH3 at about 20C, implying N  6.

The lowest k known to this writer is for uranium hexafluoride, 238UF6, which is a very complex, heavy molecule with many degrees of freedom. The estimated value of k for this heavy gas is k  1.06.